Solder Pallet Technical Specification:

| Product photo |  |  |  |  |
| --- | --- | --- | --- | --- |
| Model no. |  |  |  |  |
| Patent no. |  |  |  |  |
| keyword index |  |  |  |  |
| Application |  |  |  |  |
| Pallet ID | DIP-solder pallet-007 |  |  |  |
| Pallet Name | The solder pallet of Synthetic stone carbon fiber board for wave-soldering |  |  |  |
| Main feature | static-free ; heat-resisting material ; No deformation ; Long service life; Prevent plug-in components to float high technology |  |  |  |
| Surface treatment | Select: 1. Require（ ）; 2. Not Require（ √ ）; Process category: 1. Sandblasting & anodizing process（ ）; 2. Without sandblasting & anodizing process（ ）; 3. Teflon anodizing process（ ）; 4. Hard anodizing process（ ） Anodizing Color: 1. Original color（ ）2. Black color（ ）3. Green color （ ） 4. Pale yellow color（ ） |  |  |  |
| Application | 1. Solder paste printer: （ ）; 2. Chip Mounter: （ ）; 3. Reflow soldering: （√ ）; 4. Positioning: （ ） |  |  |  |
| Material category | 1. Synthetic stone carbon fiber board: （ √ ）; 2. Aerospace aluminum alloy: （ ）; 3. Titanium alloy material: （ ）; 4. FR4 fiberboard material: （ √ ） 5. High-Temp silicone adhesive: （ ）; 6. High-Temp Magnet: （ ）; 7. High-Temp magnetic steel 0.06mm: （ ） |  |  |  |
| Material model | Synthetic stone carbon fiber board | BLUE: 1. CDM68.650:（ ） 2. CHP760: （ ） |  |  |
|  |  | BLACK: 1. CDM-ESD68.910:（ √ ）2. CAS761: （ ） |  |  |
|  |  | GRAY : 1. CDM-ESD68.940:（ ） 2. CAG762: （ ） |  |  |
|  | Aerospace aluminum alloy | 1. 7075# : （ ） 2. 6061# : （ ） 3. 5052#: （ ） |  |  |
| Thickness | 1. 2mm（ ）2. 3mm（ ）3. 4mm（ ）4. 5mm（ ）5. 6mm（ √ ）6. 8mm（ ） |  |  |  |
| Shape & Dimension |  | 1. Pallet length L= PCB length (L1) + 60mm; 2. Pallet width W= PCB width (W1) + 60mm` 3. Specify pallet dimension: Length［ ］×Height ［ ］mm Remark: If more than one PCB board panelized in a pallet，the above calculation will be will not be applied. |  |  |
| Panelized method | Panelized method required in pallet design: 1. Yes: ( ); 2. No: (√ ) |  |  |  |
|  | Panelized method 1: （ ） | Panelized method 2: （ ） | Panelized method 3: （ ） |  |
|  | The width between PCBs: ［ ］mm，Default is 25mm |  |  |  |
|  | The number of PCB: ［ 1 ］ X ［ 2 ］panelized into one pallet |  |  |  |
| PCB angle on pallet | Longer side of PCB corresponding to the pallet track edge（ √ ） | Shorter side of PCB corresponding to the pallet track edge（ ） | PCB rotation angle（ ）Generally it can be 30 to 45 degrees |  |
| Positioning pin for holding PCB: |  | SUS304 positioning pin，which is made by CNC machine in fine processing，ensure the PCB and positioning pin at 90 degrees |  |  |
|  |  | Height of positioning pin L =PCB thickness (mm) –（0.10m～0.20mm） |  |  |
|  |  | Positioning pin diameterфX=PCB positioning hole diameterфx–0.05mm |  |  |
| Pallet heat transfer (Thermal via): Function: prevent chip components do not melt the solder paste | TOP/BOT share，avoid chip component（ ） | Create thermal via at the bottom of SMD components（ ） |  | Create thermal via at the open area of pallet（ ） |
| Wave solder pallet – open space requirement | PCB BOT component protection（ √ ） | Screw hole protection（ √ ） |  | Plastic connector pin protection（ √ ） |
| Pallet track edge dimension: |  | Pallet track in the lower part of pallet (√ ) H=2mm; W=5mm Please let us know if you have special requirements. |  |  |
|  |  | Pallet track in the upper part of pallet : H=2mm; W=5mm Please let us know if you have special requirements. |  |  |
| PCB cavity dimension |  | PCB cavity dimension = PCB dimension + 0.10mm / edge |  |  |
| PCB cavity depth dimension |  | PCB cavity depth dimension = PCB thickness – 0.10mm |  |  |
| The Packet hole for takeoff the PCB |  | In general, four packet hole will be provided, one for each direction Please let us know if you have special requirements. |  |  |
| The holddown design |  | Mainly used for SMT wave solder pallet; It includes the following accessories: high temperature plastic holddowns, springs, screws, adhesive thread locking agent |  |  |
|  |  | Mainly used for wave solder pallet(√ ) It includes the following accessories: high temperature plastic holddowns, springs, screws, adhesive thread locking agent |  |  |
|  |  | Mainly used for wave solder pallet to prevent topside component movement throughout wave solder. (√ ) It includes the following accessories: high temperature plastic holddowns, springs, screws, adhesive thread locking agent |  |  |
| Design of wave solder pallet block tin bar |  | Antistatic FR4 fiberglass board material，dimension: 10 x 10mm， 1. Black color（ √ ）; 2. white color（ ） |  |  |
|  |  | Synthetic stone materials, dimension: 6 x 6mm，black color ( ) |  |  |
|  |  | 5052# Aluminum materials, dimension: 10mm (T) x 13mm (W) |  |  |
| Pallet printing information |  | 1. WSTIANMAO LOGO （ √ ） 2. production model （√ ） 3. Date of manufacture（ √） 4. Pallet flow arrow sign （ ） 5. Serial No. （ ） 6. CAD drawing No.（ √ ） 7. ROHS marking（ √ ） 8. ESD marking （ ） Please let us know if you have special requirements. |  |  |
| Required documentation | 1. PCB board（ ）2. PCBA board（ √ ）3. PCB GERBER/CAD（ √）4. Pallet sample（ √ ） 5. CAD drawings produced by the customer（ ） 6. Special technical requirements（ ） |  |  |  |
| Production capacity | - SMT solder pallet: General processing capacity is 50 vehicles / 36 hours - DIP solder pallet: General processing capacity is 50 pallets / 48 hours - The above is the processing time within the factory and the logistics & freight time is according to the logistics company |  |  |  |
| Special requirements | 1. Prevent plug-in components to float high technology 2. To prevent the pollution flux |  |  |  |
| Remark | 1. Please insert a tick symbol “√ ” or a cross symbol“ × ”in a bracket （ ）; 2. Please put a number in a bracket ［ ］; |  |  |  |
| Technical parameters of synthetic material | All the technical parameters of this material is provided by synthetic materials manufacturers, WSTIANMAO does not have the right to decide. |  |  |  |
|  | （Vonroll Isola） Synthetic fiber technology parameters. Product number; CDM 68.650; CDM-ESD 68.910; CDM-ESD 68.940. Color; blue; black; gray. ESD; no; yes; yes. Light-reflecting; no; no; yes. Density g/cm3; 1.90; 1.90; 1.90. standard working temperature℃; 360; 360; 360. surface impedance ohms; ------; 105 ～109; 105～109. Panel size (mm); 980×1335; 1220×1950; 1335×2350. thickness of plate (mm); 3∕4 ∕ 5 ∕ 6 ∕ 8 ∕ 10 ∕ 12 ∕ 14. platness tolerance (mm); ± 0.10. （Rochling Durostone） Synthetic fiber technology parameters. Product number; CHP 760; CAS 761; CAG 762. Color; blue; black; gray. ESD; no; yes; yes. Light-reflecting; no; no; yes. Density g/cm3; 1.85; 1.85; 1.85. standard working temperature ℃; 300; 300; 300. surface impedance ohms; ------; 105 ～108; 105～108. Panel size (mm); 1220×2440. thickness of plate (mm); 3∕4 ∕ 5 ∕ 6 ∕ 8 ∕ 10 ∕ 12. platness tolerance (mm); ± 0.10. |